Sample 5th Year Engineering

Class Project

Model Motor Cycle

Developed for t4 by:

Hugh McClean

Pobalscoil Ghaoth Dobhair

Introduction:

The main parts of the project were designed to be made from 1mm sheet Aluminium, but they could also be made from Mild Steel sheet, Brass, Copper or Acrylic. The project  consists of three main parts:

Body.

Front Forks.

Handle Bars.

The project incoporates a unique double back wheel design for stability and a switch that keeps the assembly together without the need for screws.

Note:

All dimensions on the project are interdependent; if the thickness of the Aluminium is altered then the switch would have to be altered as the neck of the switch will be too short to assemble the three main parts. Likewise if the size of the gear wheel or the worm gear is altered then the position of the axle hole would also have to be altered to correspond.

All the other components used are attached with double sided adhesive pads. The motor and holder fit neatly to the underside of the main body and the two battery holders attach to the sides.

When the three profiles are completed they are assembled or joined together using the switch supplied.  This removes the need for any set screws to be used in the project.

Next the worm gear is pushed on to the 2mm Motor shaft. The Motor is placed in its holder and it is then mounted to the underside of the main body as shown in the earlier diagrams.

Assemble the rear wheels and the 38 tooth gear wheel on the 4mm Axle shaft provided.

Locate the Axle assembly in the 4mm holes in the Main body. Mount the front wheel in the Front forks in a similar manner.

The battery holders are now attached to the side of the Main body with double sided sticky pads.

The project is now wired and soldered according to either of the two following circuit diagram.

The battery may be connected in either series or parallel. In series you have 6 Volts and only 3 Volts in parallel but in parallel you will have greater torque.

Finally the design is finished with the inclusion of a backup Petrol Tank and seat. These may be moulded and or vacuum formed to a design of your own choosing.

Completed Project

Body

Handlebars

Front Fork

This project was comissioned by t4 and developed by Hugh McClean for the Leaving Certificate Technology syllabus to help students develop their craft and manufacturing skills.  The following is a possible design brief which could be used to accompany this project:

Design and manufacture a vehicle suitable for an urban and rural environment which would help to reduce to the harmful emissions currently being produced by fossil fuel vehicles.

As well as developing the students craft and technical skills the revised Leaving Certificate Engineering Technology syllabus encourages a design and make approach to project manufacture.  Therefore even in circumstances where the student may be given the final working drawings as in this situation it is important that they are made aware of the reasons why the project may have been developed.  Students could also be encouraged to decorate the completed project to their own individual design.

Pat Lynch

t4 - Assistant National Co-Ordinator ( Engineering Technology)