13. PICAXE 18X Using two ULN2803 IC drivers

By grouping the outputs of two ULN2803 transistor drivers, 8 outputs, providing one amp each, is available.

# PAXE_DD2 Component List

# SW1 = 12 x 12 mm Tactile Switch

R1 (Resistor) = 10k ohm

R2 (Resistor) = 22k ohm

R3 (Resistor) = 4k7 ohm

R4 (Resistor) = 10k ohm

R5 (Resistor) = 10k ohm

R6 (Resistor) = 10k ohm

R7 (Resistor) = 10k ohm

R8 (Resistor) = 10k ohm

CN1 = 2.1mm DC Power Socket

CN2 = 3.5mm Stereo Jack Socket

C1 (Capacitor) = 100nF (0.1uF)

C2 (Capacitor) = 220uF

C3 (Capacitor) = 220uF

C4 (Capacitor) = 100nF (0.1uF)

TB1 – TB10 = (2 way Terminal Block)

D1 (Diode) = 1N4001

D2 (Diode) = 1N4148

IC1 = 7805 Voltage Regulator

IC2 = PICAXE18 / 18A / 18x

IC3 = ULN2803 Darlington Driver

IC4 = ULN2803 Darlington Driver

ZD1 = 15V Zener Diode

PAXE_DD2 Artwork Print 120mm x 120mm