14. PICAXE 18X Multi Circuit Diagram

This circuit provides a range of inputs i.e.

- Switch

- LDR

- Temperature

- Variable resistor

Being able to detect light, heat and movement opens up a lot of possibilities in project work i.e.

- Greenhouse control

- Weather station

- Alarm

- Night light

- Thermostat control

LEDs are also used to indicate the state of each output.

# PAXE_Multi Component List

# SW1 = 12 x 12 mm Tactile Switch

R1 (Resistor) = 10k ohm

R2 (Resistor) = 22k ohm

R3 (Resistor) = 4k7 ohm

R4 (Resistor) = 10k ohm

R5 (Resistor) = 4k7 ohm

R6 - R7 (Resistor) = 10k ohm

R8 – R15 (Resistor) = 1k ohm

CN1 = 2.1mm DC Power Socket

CN2 = 3.5mm Stereo Jack Socket

C1 (Capacitor) = 100nF (0.1uF)

C2 (Capacitor) = 220uF

C3 (Capacitor) = 220uF

C4 (Capacitor) = 100nF (0.1uF)

TB1 – TB7 = (2 way Terminal Block)

D1 (Diode) = 1N4001

D2 (Diode) = 1N4148

D3 – D12  3mm LED’s

IC1 = 7805 Voltage Regulator

IC2 = DS18B20 Digital Thermometer IC

IC3 = PICAXE18 / 18A / 18x

IC4 = ULN2803 Darlington Driver

IC5 = ULN2803 Darlington Driver

ZD1 = 15V Zener Diode

LDR1 = LDR

VR1 = 100k Preset Resistor

PICAXE 18X Multi - Artwork Print 120mm x 120mm