3. PICAXE 18X, 8 LED output Circuit Diagram

8 LED output circuit: -

This circuit will allow a combination of up to 5 inputs control a combination of up to 8 LED outputs.

Suggested applications: -

- Running LED pattern

- Traffic Lights

- Pedestrian crossing

- Warning device

- Quiz control

- Binary counter

# PAXE_LED Component List

# SW1 = 12 x 12 mm Tactile Switch

R1 (Resistor) = 10k ohm

R2 (Resistor) = 22k ohm

R3 (Resistor) = 4k7 ohm

R4 (Resistor) = 10k ohm

R5 (Resistor) = 10k ohm

R6 (Resistor) = 10k ohm

R7 (Resistor) = 10k ohm

R8 (Resistor) = 10k ohm

R9 - R16 (Resistor) = 330 ohm

CN1 = 2.1mm DC Power Socket

CN2 = 3.5mm Stereo Jack Socket

C1 (Capacitor) = 100nF (0.1uF)

C2 (Capacitor) = 220uF

C3 (Capacitor) = 220uF

C4 (Capacitor) = 100nF (0.1uF)

TB1 - TB10 = (2 way Terminal Block)

D1 (Diode) = 1N4001

D2 (Diode) = 1N4148

D3 - D10 = 5mm LEDs

IC1 = 7805 Voltage Regulator

IC2 = PICAXE18 / 18A / 18x

PICAXE 18X PCB LED Artwork  - Print 120mm x 120mm